Protocol RNeasy mini kit

Preparations

- Label Eppendorf tubes for waterphase

- Prepare RNeasy mini spin column; label columns

- Make DNAse solution: 10 uL DNAse stock (fridge) + 70 uL RDD buffer (fridge) per sample (Keep on ice!!)

- Label new Eppendorf tubes for final RNA (supplied in RNeasy mini kit)

- Transfer buffers to 50 mL Greiner tubes

- Make 70% ethanol

- Turn on centrigufe to 4 °C

- Clean pipettes and workspace with RNAse-away

Protocol

- Thaw Trizol/Qiazol samples on ice

- Add 0.2 mL chloroform to samples

- Shake sample till ‘milky’ and leave at RT for 2-3 min

- Centrifuge samples for 15 minutes at 4 °C at 13000 g

- Add 350 µl of waterphase into new Eppendorf tube

- Add 350 µl (1 volume) of 70% ethanol to waterphase, mix by pipetting

- Transfer 700 µL of sample to RNeasy mini spin column (pink). Close lids and centrifuge at 20 °C for 30 sec at 8000 g. Discard flow-through

- Add 350 µL buffer RW1 to column, centrifuge for 30 sec at 8000 g. Discard flow-through.

- Add 80 µL of DNAse/RDD solution directly to membrane (Do not touch membrane). Leave at RT for 15 minutes.

- Add 350 µL buffer RW1 to column, centrifuge for 30 sec at 8000 g. Discard flow-through.

- Add 500 µL buffer RPE to column, centrifuge for 30 sec at 8000 g. Discard flow-through.

- Add 500 µL buffer RPE to column, centrifuge for 30 sec at 8000 g. Discard flow-through.

- Place RNeasy spin column in a new 2 mL collection tube. Centrifuge at 13000 g for 1 minute

- Place RNeasy spin column in 1.5 mL Eppendorf tube. Add 30 µL of RNAse-free water directly to membrane (Do not touch membrane).

- Centrifuge for 1 minute at 8000g to elute RNA.

  - When centrifuging, close collection tube lid but not Eppendorf tube: place Eppendorf tubes with lids on the same side  slowly increase centrifuge speed to prevent lids from breaking off. Critical speed is between 3000 and 6000 g, so increase slowly. Make sure to end up with 1 minute at 8000 g (so set timer to ~2.5 min to be able to increase speed).

- Remove RNeasy spin columns, keep samples on ice to measure on nanodrop

- Store samples at -80 °C for cDNA synthesis